In the name of God,

the merciful, the compassionate

# HomeWork 6
(State Feedback and PID)

By

Reza Nopour
402126924

Advanced Automatic Control
Mechanical Engineering Department
Amirkabir University of Technology
24 Dec. 23

# Problem 1 Description

# Solution

To demonstrate that if an open-loop system is controllable, then its closed-loop system is also controllable, we can use the concept of controllability matrices.

x=Ax+Bu
y=Cx

The controllability matrix, denoted as (Co), is defined as:

Co=BAB… An-1B

Now, let's consider the closed-loop system obtained by adding a feedback controller to the open-loop system. The closed-loop system can be represented as:

x=A-BKx+Bu
y=Cx

where (K) represents the feedback gain matrix. To show that the closed-loop system is also controllable, we need to examine the controllability matrix (Cc) for the closed-loop system, defined as:

Co=BA-BKB… A-BKn-1B

BAB… An-1B=BA-BKB… A-BKn-1BQ

in which the determinant of Q matrix is one and shows the controllability matrix of two systems (open and closed) are the same!

Q=1kb…kAn-1b01…⋮00⋱⋮00…1

b) **If part (open-loop observable => closed-loop minimal and observable):**

If the open-loop system is observable, then the observability matrix (O) has full rank. Now, let's consider the closed-loop system. If it is minimal (no pole-zero cancellation), then (Ac) is full rank. Since (Cc) is the same as (C) in this case, the observability matrix (Oc) is formed by stacking the matrices (C), (CAc), (CAc2), ..., (CAcn-1). Since (Ac) is full rank, the observability matrix (O_c) also has full rank, and thus the closed-loop system is observable.

**Only if part (closed-loop minimal and observable => open-loop observable):**

Conversely, if the closed-loop system is minimal and observable, then the observability matrix (Oc) has full rank. Now, we can express (Ac) in terms of the open-loop matrices:

[ Ac = A - BK = A - BKK-1 = A - AcK-1 ]

Now, the observability matrix (Oc) is formed by stacking the matrices (C), (CAc), (CAc2), ..., (CAcn-1). Since (Oc) has full rank, it implies that the matrices (C), (CAc), (CAc2), ..., (CAcn-1) are linearly independent.

Consider (CAci). If (CAci) is linearly independent, then (CAi-1) (which is a submatrix of (O)) must also be linearly independent. Therefore, the observability matrix (O) has full rank, and the open-loop system is observable.

In conclusion, if the open-loop system is observable, then the closed-loop system is observable if and only if it is minimal.

# Problem 2 Description

# Solution

Considering the position of M as x instead of y according to the illustration.

T=12Mx2+12mx2+l2θ2+2xlθcosθ
U=mglcosθ

So, the Lagrangian form of this system can be written as,

∂T∂x=Mx+mx+lθcosθ
ddt∂T∂x=Mx+mx+lθcosθ-lθ2sinθ
∂T∂θ=mxlcosθ+l2θ
ddt∂T∂θ=mxlcosθ+l2θ-xθlsinθ
 ∂T∂x=0, ∂T∂θ=-mxθlsinθ
∂U∂x=0, ∂U∂θ=-mglsinθ
Qx=f, Qθ=0.

The system equation of motion can be derived as,

Mx+mx+lθcosθ-lθ2sinθ=fmxlcosθ+l2θ-xθlsinθ+mxθlsinθ-mglsinθ=0

Above equations are nonlinear, considering small rotation, the equation can be rewritten as,

Mx+mx+lθ-lθ2θ=f                  *x+lθ-gθ=0→ θ=gθ-xl     **

Substituting θ in * yields,

Mx+mx+gθ-x-lθ2θ=f→ x+mMgθ-lθ2θ=fM

it can be derived as,

x=fM-mgMθ-mlMθ2θ

Obtained x can be inserted into **,

θ-M+mMlgθ-mMθ2θ=-fMl

Final equation can be obtained as,

x+mgMθ=fM                  ***θ-M+mMlgθ=-fMl     ****

According to the question,

τ=rf
rθ=x→rω=x
→f=τr=kRre-k2Rrω=kRre-k2Rr2x

thus,

x+k2MRr2x+mgMθ=kMRre                  *θ-k2MlRr2x-M+mMlgθ=-kMlRre     **

State Space representation of this system is,

x=01000-k2MRr2-mgM000010k2MlRr2M+mMlg0xxθθ+0kMRr0-kMlRre
y=10000010x+00e

where,

m=0.1 kg, M=1.0 kg, l=0.1 m, g=9.8 m.s-2, k=1 V.s, R=100 Ω, r=0.02 m

A=01000-25-0.980000102510.780, B=00.50-0.5

According to the Ackerman Formula:

kT=0001Q-1ΔcA

Δcs=s+2-j23s+2+j23s+4s+25=s4+33s3+232s2+864s+1600
ΔcA=A4+33A3+232A2+864A+1600
Q=controllability Matrix=BABA2BA3B

Q=00.5-12.53130.5-12.5312.99-78370-0.512.5-317.9-0.512.5-317.897959.5
Q-1=502.200.22.2-5.1020.2-5.102-5.102-0.204-5.102-0.204-0.2040-0.2040

k=-326.5306-226.3265-812.0906-242.3265

- Designing a PID controller.

PID:C=P+I1s+Ds=Ds2+Ps+Is

For a closed-loop system,

UYPID=CG1+CG

For a closed-loop system,

UYsf=G1+GH

Evaluating G, or the transfer function of the original system as,

CsI-A-1B+D=10000010s0000s0000s0000s-01000-25-0.980000102510.780-100.50-0.5→
G=-0.5s2s4+25s3-10.78s2-245s→G=-0.5ss3+25s2-10.78s-245

PID for default system:

Using Routh stability criterion for PID controller, in order to place the poles of the system in the LHP.

| Routh Criterion |  |  |
| --- | --- | --- |
| s3 | 1 | -10.78-0.5P |
| s2 | 25-0.5D | 245-0.5I |
| s | 25-0.5D-10.78-0.5P-(-245-0.5I)25-0.5D |  |
| 1 | -245-0.5I |  |

GC=-0.5Ps+Ds2+Is3+25-0.5Ds2-10.78+0.5Ps-245-0.5I

Therefore,

25-0.5D>0→0.5D<25→D<50→D=40
-245-0.5I>0→0.5I<-245→I<-490→I=-500
25-0.5D-10.78-0.5P-(-245-0.5I)25-0.5D>0→P<-23.56→P=-35

PID for the shifted system:

The system poles are,

s+2-j23s+2+j23s+4s+25=s4+33s3+232s2+864s+1600
polese located at -25, can be ignored as it is far away from the origin and others.

s+2-j23s+2+j23s+4s+25=s3+8s2+32s+64

and

GC=-0.5Ps+Ds2+Is3+25-0.5Ds2-10.78+0.5Ps-245-0.5I

therefore,

25-0.5D=8→D=34 
-10.78+0.5P=32→P=-85.56
-245-0.5I=64→I=-618

The answers satisfy the condition of the default Routh criterion,

# Problem 3 Description

# Solution

- The controllability matrix is as follows,

Q=BAB=-1-111
Q=0→Uncontrollable

PBH method in finding uncontrollable mode,

vTQ=0

The eigenvalue of the system is equal to zero, and corresponding eigenvector is,

v=11

11-1-111=0→uncontrollable mode

It is marginally stable.

- Observability matrix can be written as,

O=CCA=0c20-c2
O=0→ it is not observable

- Rewriting the state space equation,

x=010-1x--11k1k2x+-11uex=k11+k2-k1-1-k2x+-11uex

- Forming the controllability matrix,

Q=BAB=-1-k1+k2+11k1-k2-1
Q=0→uncontrollable

According to the PBH test, uncontrollable mode is,

vTA=λvT

vTQ=0 
v=11
11-11=0
Similar to part a.

- CA=0c2k11+k2-k1-1-k2=-k1c2-c2-k2c2

Observability matrix can be written as,

O=CCA=0c2-k1c2-c2-k2c2
O=-k1c22≠0→k1≠0

For k1≠0, it can be observable.

- The transfer function of the system can be written as,

CsI-A-1B+D=c1c2s00s-k11+k2-k1-1-k2-1-11=c1c2s2+1-k1+k2ss+1+k21+k2-k1s-k1-11=s(c2-c1)s2+1-k1+k2s=s(c2-c1)ss+1-k1+k2

It can be observed that always zero-pole cancellation for s=0, happens in this system and systems remains uncontrollable.

# Problem 4 Description

# Solution

The transfer function can be obtained as,

Gs=-2(0.1s+1)(s+1)ss-22s+12

the canonical controllable form can be written as,

x1x2x3x4x5=010000010000010000010-4-432x1x2x3x4x5+00001u
y=-2-2.2-0.200x

therefore, controllability matrix can be written as,

Co=BABA2BA3BA4B=0000100012001270127161271641→Co≠0→full rank→controllable

detsI-A=s00000s00000s00000s00000s-010000010000010000010-4-432=s-10000s-10000s-10000s-1044-3s-2=s5-2s4-3s3+4s2+4s

a=-2-3440
pf=-5+2i-5-2i-10-15-20
ps=-0.1-1-0.5i-1+0.5i-2-5

Ψ=1-2-34401-2-34001-2-30001-200001

Bass-Gura,

pf=551129108054885087000
ps=9.126.1531.27515.37501.25

kT=αf-aCo×Ψ-1= 5711321080148846870000000100010001000100010000=870004884610801113257

kT=αs-aCo×Ψ-1= 11.129.1527.27511.3751.250000100010001000100010000=1.2511.37527.27529.1511.1

Ackermann,

kT=00001Co-1pdfA=0000144-3-214-3-210-3-2100-210001000087000I+484850A+10805A2+1129A3+55A4+A5=10000870004884610801113257086772486181097212460-4984 8178852356134640-53856-58840122180792840-317136 -370992 179012280748=870004884610801113257

kT=00001Co-1pdfA=0000144-3-214-3-210-3-2100-21000100001.25I+15.375A+31.275A2+26.15A3+9.1A4+A5=100001.2511.37527.27529.1511.10-43.15-33.02560.57551.350-205.4 -248.55121.02163.270-653.1-858.5241.27447.580-1790.3 -2443.4 484.221136.4=1.2511.37527.27529.1511.1

Mayne-Murdoch,

:..)

# Problem 5 Description

# Solution

Check controllability of the system,

Q=BAB=011-10
Q=-1≠0→full rank
the system is controllable and now we can design a controller.

According to the given information,

ts<1 s→3ζωn<1→ζωn>3  5% criteria

Finding ζ

Mp<0.25→e-ζ1-ζ2π<0.25→ζπ1-ζ2>1.386→ζ>0.40
→ζωn>3→0.4ωn>3→ωn>7.5

Considering ωn=8, and ζ=0.8

For a closed-loop system we have,

s2+2ζωns+ωn2

which becomes,

Δd=s2+12.8s+64

In the direct method,

u=-k1k2x→A-BK=01-k1-10-k2

Calculating the determinant of the sI-A-BK,

sI-A-BK=s-1k1s+10+k2=s2+10+k2s+k1

Δd=s2+10+k2s+k1
k1=64
k2=2.8

Verifying the results with Ackermann!

k=01Q-1ΔdA
k=0110110A2+12.8A+64I=0110110642.8036=642.8

# Problem 2.1 Description

# Solution

The system is implemented in SIMULINK and the results is as follows,

Comparing control efforts is plotted here,

If the square of the signal integral be used, PID controller has much control effort than feedback gain ones,

For comparing θ, and x we have,

Figure 1 θ, and x during a 10 seconds simulation.

It is apparent that the PID controller possesses the capability to stabilize the system. However, due to the lack of feedback from the position signal of the cart, it is unable to regulate the cart's position effectively, resulting in persistent steady-state error. Furthermore, although the overshoot generated by the PID controller is less than that of feedback controllers, its response speed is significantly lower compared to feedback controllers. Furthermore, observations reveal that linear controllers have provided relatively satisfactory responses for nonlinear equations. Considering that the initial angle of the pendulum is set to 10 degrees, linearized equations approximate the same output as the nonlinear equations. Consequently, linear controllers have proven effective for nonlinear equations as well.

# Problem 2.2 Description

# Solution

Codes have been written in MATLAB and are as follows,

clc clear close all syms s p_f=[-5-2i -5+2i -10 -15 -20]; pf=poly(p_f); pf(1)=[]; p_s=[-0.1 -1-0.5i -1+0.5i -2 -5]; ps=poly(p_s); ps(1)=[]; A=[0 1 0 0 0;0 0 1 0 0;0 0 0 1 0;0 0 0 0 1;0 -4 -4 3 2]; B=[0;0;0;0;1]; C=[-2 -2.2 -0.2 0 0]; D=[0]; [b,aa]=ss2tf(A,B,C,D) Co=ctrb(A,B) fprintf('the rank of controllability matrix is %d. \n',rank(Co)) if rank(Co)==length(Co) disp('The matrix is full rank and the system is controllable!') else disp('The matrix is not full rank and the system is uncontrollable!') end Ob=obsv(A,C) fprintf('The rank of observability matrix is %d. \n',rank(Ob)) if rank(Ob)==length(Ob) disp('The matrix is full rank and the system is Observable!') else disp('The matrix is not full rank and the system is not observable!') end

The results can be seen as follows,

Co = 0 0 0 0 1 0 0 0 1 2 0 0 1 2 7 0 1 2 7 16 1 2 7 16 41 the rank of controllability matrix is 5. The matrix is full rank and the system is controllable! Ob = -2.0000 -2.2000 -0.2000 0 0 0 -2.0000 -2.2000 -0.2000 0 0 0 -2.0000 -2.2000 -0.2000 0 0.8000 0.8000 -2.6000 -2.6000 0 10.4000 11.2000 -7.0000 -7.8000 The rank of observability matrix is 4. The matrix is not full rank and the system is not observable!

It can be understood that results are same as the ones obtained in the first part. The system is controllable but not observable. To find out the unobservable mode, obsvf(A,B,C) is used.

[Abar,Bbar,Cbar,T,K]=obsvf(A,B,C)

And the response is

Abar = -1.0000 -0.8519 -1.5859 2.1583 1.1406 0.0000 1.8869 2.8052 -3.7973 -3.0389 0.0000 1.1449 0.3726 -0.5111 -0.0692 -0.0000 0.0000 0.9335 0.1954 -0.3006 0.0000 -0.0000 -0.0000 0.8384 0.5450 Bbar = 0.4472 -0.8903 -0.0858 0 0 Cbar = 0.0000 -0.0000 -0.0000 0.0000 2.9799 T = 0.4472 -0.4472 0.4472 -0.4472 0.4472 0.2543 -0.2532 0.2429 -0.1399 -0.8903 -0.3075 0.2968 -0.1898 -0.8798 -0.0858 0.4363 -0.3206 -0.8369 -0.0801 0 -0.6712 -0.7383 -0.0671 0 0 K = 1 1 1 1 0

The results show the mode corresponding to eigen -1 is unobservable.

- Results of parts i,ii,iii can be obtained and verified as follows,

a=[-2 -3 4 4 0]; Psi=[1 a(1) a(2) a(3) a(4) 0 1 a(1) a(2) a(3) 0 0 1 a(1) a(2) 0 0 0 1 a(1) 0 0 0 0 1 ]; %% Fast k_bass_fast=(pf-a)*inv(Co*Psi) k_acker_fast=acker(A,B,p_f) k_acker_fast2=[0 0 0 0 1]*inv(Co)*(pf(5)*eye(5)+pf(4)*A+pf(3)*A^2+pf(2)*A^3+pf(1)*A^4+A^5) %% Slow k_bass_slow=(ps-a)*inv(Co*Psi) k_acker_slow=acker(A,B,p_s) k_acker_slow2=[0 0 0 0 1]*inv(Co)*(ps(5)*eye(5)+ps(4)*A+ps(3)*A^2+ps(2)*A^3+ps(1)*A^4+A^5)

Results are same as those calculated in part I.

k_bass_fast = 87000 48846 10801 1132 57 k_acker_fast = 87000 48846 10801 1132 57 k_acker_fast2 = 1.0e+04 * 8.7000 4.8846 1.0801 0.1132 0.0057 k_bass_slow = 1.2500 11.3750 27.2750 29.1500 11.1000 k_acker_slow = 1.2500 11.3750 27.2750 29.1500 11.1000 k_acker_slow2 = 1.2500 11.3750 27.2750 29.1500 11.1000

- Slow and fast responses as well as control effort are plotted in this part,

Figure 2 Fast response of the system.

Figure 3 Slow response of the system.

Figure 4 Control effort for fast response of the system.

Figure 5 Control effort for slow response of the system.

The control effort required for achieving a fast response in the system is significantly higher compared to achieving a slower response. Additionally, it is evident that by utilizing fast modes, the system can reach a stable state more quickly than when relying on slower modes.

- For part 1.Q5 we have:

%% Problem 5 A=[0 1; 0 -10]; B=[0;1]; C=[1 0]; Poles=roots([1 12.8 64]).'; k=place(A,B,Poles)

which the answer is as follows,

k = 64.0000 2.8000

which is exactly what we have obtained in part I.